EV Charging Station Maintenance

Optimizing Technician Dispatch — A Longitudinal Analysis

# Executive Summary

To be written last, once all analyses are complete.

# 1. Background

A network of 638 electric-vehicle charging stations is currently operated under a uniform maintenance policy: a technician visits every station every two to four weeks, regardless of how heavily the station is used or what time of year it is.

The operator wants to know whether this fixed cycle should be adapted. Two operational hypotheses are on the table:

- Utilization-based scheduling. Stations with heavier traffic may wear out faster and warrant more frequent visits, while quieter stations could be safely visited less often.

- Seasonal scheduling. Failure rates may rise in spring or summer due to higher usage and weather stress, justifying a tighter cycle in those months.

The goal is twofold: maintain network reliability (few incidents) while reducing unnecessary technician deployments. This report analyzes six months of operational data to evaluate both hypotheses.

# 2. Data

## 2.1 Source and structure

The dataset covers 638 charging locations over six months (January to June 2024) and is organized across three data sheets at the (location, month) level:

- Sessions records charging activity per location and month, including the number of charging points (evse_count), the total number of charging sessions (session_count), the per-charging-point session count (sessions_per_evse), and the average session duration in seconds.

- Tasks records the number of completed technician visits per location and month (tasks_solved). A task corresponds to a scheduled inspection visit during which the technician checks the equipment.

- Incidents records the number of defects documented during these visits per location and month (incidents_created). An incident is a noticeable hardware issue — for example, a defective plug or a broken display — observed and logged by the technician during a task.

A key structural property of the data: incidents can only be reported during a task. There is no mechanism for an incident to be recorded outside of a technician visit. This dependency between tasks and incidents is central to the analytical framing developed in Section 3.

## 2.2 Coverage and missing data

Coverage of the three sheets is uneven. The Sessions sheet is the most complete: charging activity was recorded for 3,661 (location, month) cells across the six-month window, covering all 638 unique locations. The Tasks sheet contains 2,407 records covering 572 locations, meaning approximately 35% of (location, month) cells with charging activity have no associated technician visit recorded. The Incidents sheet contains 1,040 records covering 454 locations, meaning approximately 72% of cells have no incident record.

| Sheet | Rows | Unique locations | Cell-level coverage |
| --- | --- | --- | --- |
| Sessions | 3,661 | 638 | 100% |
| Tasks | 2,407 | 572 | 65.4% |
| Incidents | 1,040 | 454 | 28.1% |

Table 1. Recording coverage across the three data sheets.

The monthly breakdown reveals a notable operational pattern: the number of recorded technician visits rises substantially over the period, from 302 in January to 482 in June. This 60% increase is unlikely to reflect a sudden change in maintenance policy and more plausibly indicates a ramp-up in operational data recording during the early months of the year. This pattern matters when interpreting January results in the seasonality analysis (Section 5.2).

Figure 1. Number of (location, month) cells with a recorded technician visit, by month.

The interpretation of missing values in the Incidents sheet requires care. A (location, month) cell with no incident record can mean either that the technician visited and found no defects, or that the visit did not happen and no record was created. These two interpretations lead to materially different incident-rate estimates. To address this ambiguity, all analyses are run under both assumptions:

- Missing as zero: where a technician visit was recorded but no incident row exists, we treat the incident count as zero. Under this assumption, the analysis sample contains 2,396 (location, month) observations with a mean incident rate of approximately 10%.

- Missing as unknown: rows with missing incident or task values are excluded entirely. This conservative subset contains 1,030 observations with a mean incident rate of approximately 24%.

The primary analysis uses the missing as zero assumption, which aligns with the operator’s data-collection process: every station is on a fixed inspection cycle, so the standard outcome of a visit is "no incident found." The missing as unknown assumption is used as a robustness check in Section 6 to confirm that conclusions do not depend on this choice.

## 2.3 Descriptive statistics

The following table summarizes the key variables on the analysis sample (2,396 (location, month) cells under the missing-as-zero assumption). Three properties of the data shape the modelling decisions in the next section: utilization is positive and right-skewed, the count variables (tasks_solved, incidents_created) are bounded below by zero with long upper tails, and the outcome of interest — the incident rate — has a heavy mass at zero.

| Variable | Mean | Median | Std | Min | Max | % zeros |
| --- | --- | --- | --- | --- | --- | --- |
| utilization (sessions/EVSE) | 155.3 | 123.2 | 111.2 | 1.5 | 704.7 | 0% |
| session_count | 959.8 | 801.0 | 716.3 | 3.0 | 6510.0 | 0% |
| evse_count | 6.7 | 6.0 | 3.4 | 1.0 | 27.0 | 0% |
| tasks_solved | 11.9 | 12.0 | 8.5 | 1.0 | 72.0 | 0% |
| incidents_created | 1.0 | 0.0 | 1.8 | 0.0 | 25.0 | 57% |
| incident_rate | 0.10 | 0.00 | 0.24 | 0.00 | 6.00 | 57% |

Table 2. Descriptive statistics for the analysis sample (n = 2,396).

The distributions of the four variables that will enter the model are shown below. Utilization, plotted on a log scale, is approximately log-normal — bell-shaped on the log axis. Tasks_solved is roughly unimodal around 12 visits per location-month, with a long tail. Incidents_created and the incident rate are dominated by zero values: 57% of observations have no incident at all, with the rest concentrated below 0.4 incidents per task and a small handful of outliers.

Figure 2. Distributions of the four key modelling variables.

Two consequences for the analysis follow directly. First, because utilization is log-normal, the joint model in Section 5.3 uses log(utilization) rather than the raw value — this normalizes the scale and makes the multiplicative interpretation natural. Second, because the incident rate is zero-heavy, we use a Poisson-family model rather than a Gaussian regression: Poisson is well-suited to non-negative count outcomes with concentrations at zero.

# 3. Analytical Framing

The most important methodological choice in this analysis concerns the unit of analysis. The intuitive approach to the operator’s question is to model the raw number of incidents as a function of utilization: do busier stations log more incidents? But this approach has a subtle confound that, if uncorrected, leads to misleading conclusions.

The confound. An incident can only be recorded during a technician visit. The number of incidents at a given station in a given month is therefore the product of two factors:

incidents = tasks × (incidents per task)

A regression of raw incident counts on utilization (or on tasks) is unable to separate these two factors. A positive coefficient may reflect either that busy stations actually fail more (the question of interest) or simply that busy stations are inspected more thoroughly and therefore accumulate more recorded defects (a mechanical artefact of the inspection process). The two are statistically indistinguishable in a model that does not condition on inspection volume.

The fix. Throughout this report, the unit of analysis is the incident rate — the number of incidents found per technician visit. The rate answers the operator’s actual question directly: given that we visit a station, how often do we find something broken? Modelling the rate isolates station condition from inspection frequency and removes the confound described above.

The descriptive picture already strongly suggests where the answer will land. The figure below plots the incident rate against utilization, with a LOWESS smoother fit through all 2,396 observations. The smoother is essentially flat: across more than two orders of magnitude in utilization (from a handful of sessions per charging point to several hundred), the local mean rate stays between 2% and 5%. There is no visible monotone trend.

Figure 3. Utilization vs incident rate, with LOWESS smoother. The flat smoother at the descriptive level previews the formal result of the joint model in Section 5.3.

This is the descriptive precursor of the result that the joint model in Section 5.3 will quantify with confidence intervals. Among station-month observations there is no evidence that higher utilization is associated with a higher per-visit failure rate — the operator’s first hypothesis is unlikely to be supported.

A note on zero-inflation. The incident rate has a 57% zero mass. We retain a standard Poisson family for the joint model rather than moving to a zero-inflated variant: at the very low mean rate observed here (roughly 0.10), Poisson naturally produces a high zero probability and the simpler family is sufficient. We test this assumption explicitly via residual diagnostics in Section 6 — if overdispersion is detected, the appropriate next step would be a negative binomial generalization rather than zero-inflation.

# 4. Methods

Three complementary statistical approaches are used to evaluate the operator’s two hypotheses. Each is chosen for its fit with the structure of the data — non-negative count outcomes, repeated measures within station, and a heavily skewed predictor.

## 4.1 Tier comparison (utilization hypothesis)

Stations are grouped into three utilization tiers (Low / Medium / High) using tertiles of their per-station mean utilization across the six observed months. Tertiles, rather than raw values or quartiles, map naturally onto the operator’s decision lever: a three-speed inspection cycle (longer / standard / shorter). Per-station means are used rather than per-row utilization so that a single physical station does not move between tiers month to month — the tier itself becomes a stable property of the station.

Differences in incident rate across tiers are tested with the Kruskal-Wallis rank test, a non-parametric ANOVA-equivalent that does not assume normality and is robust to the outliers present in the data. A complementary station-level Spearman rank correlation between mean utilization and mean incident rate provides a direct test of monotone trend.

## 4.2 Seasonality test

Differences in incident rate across the six calendar months are tested with the same Kruskal-Wallis framework, applied to the per-month distributions of incident rate. The test treats the rate as the unit of comparison — not the raw incident count — so the result is not confounded by month-to-month differences in inspection volume.

## 4.3 Joint Poisson model

The descriptive tests above are univariate. To estimate the utilization and seasonality effects simultaneously while accounting for repeated measures within station, the analysis uses a generalized estimating equations (GEE) model with a Poisson family.

The model specification is:

log E[incidents] = β₀ + β₁ · log(utilization) + γ · month + log(tasks)

Three details make this model the right tool for the job:

- log(tasks) enters as an offset, not a covariate. Forcing its coefficient to one converts the regression of expected counts into a regression of expected rates per task. The exponentiated coefficients on the other variables are therefore directly interpretable as multiplicative effects on the incident-per-visit rate, which is the quantity of operational interest.

- GEE rather than mixed effects. GEE estimates population-averaged effects (the average rate ratio across the network), which corresponds to the operator’s decision question. A station-level mixed-effects (subject-specific) interpretation would be less direct. GEE is also more robust to misspecification of the within-station correlation structure.

- Exchangeable working correlation. Observations within a station are treated as equally correlated regardless of the time gap between months — a defensible default for a six-month panel where a richer time structure is not separately identifiable from the data.

Coefficients are exponentiated to rate ratios, with 95% confidence intervals from robust ("sandwich") standard errors. A rate ratio of 1.0 indicates no effect on the incident rate; values above 1 indicate elevation, below 1 indicate reduction. Two robustness checks (Section 6) re-fit the model under the alternative missing-data assumption and assess Poisson distributional fit via the dispersion ratio of Pearson residuals.

| Method | Where used | What it answers |
| --- | --- | --- |
| Kruskal-Wallis test | Sections 5.1, 5.2 | Do incident-rate distributions differ across utilization tiers, or across calendar months? |
| Spearman rank correlation | Section 5.1 | Is there a monotone relationship between station-level utilization and incident rate? |
| Bootstrap (1000 resamples) | Section 5.1 | How precise are the per-tier mean estimates? (95% CIs) |
| GEE Poisson with offset | Section 5.3 | What are the rate-ratio effects of utilization and seasonality on the incident rate, controlling for repeated measures within station? |

Table 4. Statistical methods used in the analysis at a glance.

# 5. Results

## 5.1 Utilization analysis

This section evaluates the operator’s first hypothesis: do stations with heavier traffic have higher per-visit incident rates than quieter ones? The analysis aggregates each of the 571 stations to its mean monthly utilization and mean incident rate, groups stations into three utilization tiers using tertiles of mean utilization, and tests for differences across tiers.

| Tier | n | Mean rate | Median | Std | 95% CI of mean | Utilization range |
| --- | --- | --- | --- | --- | --- | --- |
| Low | 191 | 0.103 | 0.056 | 0.157 | [0.082, 0.127] | 3.3 – 82.6 sess/EVSE |
| Medium | 190 | 0.126 | 0.083 | 0.181 | [0.101, 0.152] | 82.8 – 161.4 |
| High | 190 | 0.088 | 0.075 | 0.086 | [0.076, 0.100] | 162.3 – 694.8 |

Table 3. Mean incident rate by utilization tier, with 95% bootstrap confidence intervals (n = 571 stations).

The pattern is not what the hypothesis predicts. Under "more usage means more wear-and-tear," the mean incident rate should rise from Low to Medium to High. Instead, the High tier — the most heavily-used stations — has the lowest mean rate (0.088) and is also the most consistent across stations (standard deviation 0.086 vs. 0.157–0.181 for Low and Medium). Medium-tier stations show a slight elevation (0.126), but the 95% confidence intervals overlap substantially with both other tiers, so this difference is not stable.

Figure 4. Mean incident rate per tier with 95% bootstrap CIs (left) and the full station-level distribution per tier (right). The Low and Medium tiers are right-skewed; the High tier is more compact.

Two formal tests confirm the visual reading:

| Test | Statistic | p-value | Interpretation |
| --- | --- | --- | --- |
| Kruskal-Wallis (across tiers) | H = 5.21 | 0.074 | Not significant at α = 0.05 |
| Spearman (utilization vs rate) | ρ = 0.038 | 0.369 | No monotone trend |

Table 5. Statistical tests of the utilization hypothesis.

Neither test detects an effect. The Kruskal-Wallis test does not reject equality of distributions across tiers at the 5% level. The Spearman rank correlation — which directly tests for a monotone trend at the station level — is essentially zero, with a p-value far from significance.

Conclusion. The data do not support utilization-based scheduling. Across two orders of magnitude in utilization (from 3 to nearly 700 sessions per charging point), the per-visit incident rate is essentially flat. Heavily-used stations are not more failure-prone than quieter ones — if anything, the High tier is slightly more reliable and more consistent. This descriptive finding is consistent with the LOWESS smoother shown in Section 3 and will be confirmed with confidence intervals by the joint Poisson model in Section 5.3.

A note on the High-tier consistency. The substantially lower variance among heavily-used stations (std = 0.086 vs. 0.16–0.18 for the other tiers) is suggestive of a maintenance regime that has converged on these stations. One plausible explanation is that high-traffic stations attract more operational attention — more frequent informal checks, faster response to user complaints, dedicated technicians — producing both lower failure rates and tighter station-to-station variability. The data do not let us test this directly; it is offered as a hypothesis worth following up rather than a finding.

## 5.2 Seasonality analysis

Phase 4 — to be filled after the seasonality analysis is complete.

## 5.3 Joint model

Phase 5 — to be filled after the GEE Poisson model is fit.

# 6. Robustness Checks

To be written after the joint model is complete.

# 7. Recommendations

To be written near the end, once all results are in.

# 8. Limitations

To be written near the end. Will note that 9 of 2,396 observations have rate > 1 (multiple defects per visit), retained in the analysis as operationally explainable; and that the spring elevation observed in seasonality cannot be confirmed as recurring without a second year of data.

# Methods Appendix

Technical details of statistical methods, written progressively as each phase is completed.